# Power Factor Analyser – Project Logbook

| Student Name: | Joseph Westing |
| --- | --- |
| Student number: | 24026929 |
| Module: | UGMFGT_15_1 |
| Subject: | Programming for Engineers |
| Project: | Power Factor Analyser |
| Tutors: | Ghaly Mikhail, Tom Knowles, Paul Worgan |

Contents

1.	Design	2

2.	Development	2

3.	Testing	7

4.	Reflection	8

# Design

  - Struct

The struct was designed to represent a single row of the CSV file, with each field mapping directly to a column in the dataset. This makes the data self-contained and easy to process as discrete samples rather than raw text.

Each field was stored as a float because the input data contains decimal values and requires continuous numerical precision for signal processing. The struct approach also makes it easier to pass grouped data between functions without handling multiple parallel arrays, reducing complexity and improving readability.

What was intentionally not included was any processing logic inside the struct itself. The struct was kept as a pure data container to maintain separation between data and computation, making the design more modular and reusable.

  - Pointers

Pointers were used to allow dynamic handling of the dataset rather than relying on fixed-size arrays. The main dataset is accessed through a pointer to a dynamically allocated block of memory containing multiple struct instances.

Pointer arithmetic is used when iterating through the dataset. For example, incrementing the pointer moves to the next struct in memory (ptr + i), allowing sequential access without needing array indexing in some parts of the code.

This approach was chosen because it scales with dataset size and avoids stack memory limitations. It also allows functions to operate directly on memory addresses, improving efficiency.

  - Memory Management

Memory for the dataset was allocated using malloc, which reserves a continuous block of heap memory based on the number of samples required:

- The allocation size is calculated as number_of_samples * sizeof(struct Power_Log)

- malloc returns a pointer to the first byte of this memory block

- This pointer is then treated as an array of structs

If free() were removed, the program would leak memory because the allocated heap space would not be released back to the system after use. Over time, this would increase memory usage unnecessarily and could eventually lead to performance degradation or crashes if the program were scaled or run repeatedly.

The free() call ensures proper cleanup once processing is complete, maintaining predictable memory behaviour and preventing resource accumulation.

# Development

  - 26/03/26: Start of Development

    - Explanation

Initial project files were created within the CLion IDE. The directory was set up using OneDrive for cloud access. GitHub was also set up to allow for commit and push to repository.

    - Problems Encountered

Some issues arose while trying to set up the GitHub repository and pushing from the IDE. After some trial and error, it turned out to be a dyssynchronization between the online repository and the local repository.

  - 09/04/26: Power Log Struct

    - Explanation

typedef struct {
    double timestamp;
    double phaseA;
    double phaseB;
    double phaseC;
    double lineCurrent;
    double frequency;
    double powerFactor;
    double thd;
} Power_Log;

The struct was created containing float values for each of the headers in the CSV file. The variables had to be floats because the expected values contained decimal points. The struct represents a single line of the CSV file. So the program creates 1000 of these structs to store the full dataset.

    - Problems Encountered

Only small issue at this point was that commas were used instead of semi colons. After checking the syntax in the course material, this change was implemented and the IDE no longer flagged an error.

  - 11/04/26: Struct Header

    - Explanation

The Power_Log struct was moved to a new header file called Power_Log_Extraction.h. The purpose of this move was to keep the elements of this project modular. That way, this header file could potentially be picked up and used in another project.

    - Problems Encountered

n/a

  - 11/04/26: CSV Extraction

    - Explanation

A new header file was introduced to manage CSV file operations. This included function prototypes responsible for opening the file, reading line-by-line data, and populating the Power_Log struct array. The extraction logic reads each row from the CSV, splits values based on delimiters, and assigns them to the corresponding struct fields. This allowed the dataset to be imported into memory for further processing.

    - Problems Encountered

Initial parsing attempts incorrectly handled delimiter separation, causing misaligned data fields. This was resolved by verifying CSV formatting and adjusting the string to correctly match column structure.

  - 11/04/26: Dynamic Memory (Malloc)

    - Explanation

The dataset storage was redesigned to use dynamically allocated memory instead of fixed-size arrays. Memory was allocated using pointers to allow scalability and improved flexibility depending on dataset size. This also reduced memory waste and enabled safer handling of larger datasets. Pointer-based access was implemented throughout the CSV loading and processing functions.

    - Problems Encountered

Initial implementation caused segmentation faults due to incorrect memory allocation size calculations. This was resolved by ensuring allocation was based on sizeof(struct) multiplied by dataset length.

  - 20/04/26: RMS

    - Explanation

Added capability to calculate all three phase RMS values. These were stored in a new struct:

typedef struct {
    double rms_phaseA;
    double rms_phaseB;
    double rms_phaseC;
    double rms_current;
} RMS_Result;

The code takes each value in the logs for a single phase, squares them, adds them all up, then divides by the number of entries, then finds the square root. The resultant value is the RMS. This process is repeated for each phase.

    - Problems Encountered

  - 20/04/26: io.c / io.h

    - Explanation

File handling and data ingestion functions were moved into io.c and declared in io.h. This improved modularity and separation of concerns within the project. It also made the codebase easier to maintain and allowed reuse of I/O functions across other modules.

    - Problems Encountered

Minor linking issues occurred due to missing function declarations in the header file. These were resolved by ensuring all function prototypes were correctly defined in io.h.

  - 20/04/26: Peak to Peak & DC Offset

    - Explanation

Additional signal processing functions were added to compute peak-to-peak values by identifying maximum and minimum values within each dataset. DC offset was calculated by averaging all samples within each phase. These metrics were added to the analysis struct alongside RMS values.

    - Problems Encountered

Edge cases with negative values initially caused incorrect peak detection. This was resolved by ensuring proper initialisation of min/max tracking variables.

  - 21/04/26: Restructure into Directories

    - Explanation

Source files were reorganised into logical directories (e.g., waveform analysis, I/O handling). This improved navigation and separated functional components of the system. CMake configuration was updated accordingly to reflect new file paths.

    - Problems Encountered

Build errors occurred due to incorrect CMake paths after restructuring. These were resolved by updating the add_executable file list.

  - 21/04/26: Clip Detection

    - Explanation

A function was introduced to detect waveform clipping by checking if values exceed predefined voltage thresholds. If clipping is detected, a flag is set within the analysis struct. This allows identification of signal distortion within the dataset.

    - Problems Encountered

False positives occurred due to incorrectly set threshold values. These were corrected after reviewing expected signal ranges.

  - 27/04/26: CSV Filepath

    - Explanation

The file path to the CSV dataset was corrected to match the updated project directory structure. This ensured consistent file access regardless of build environment or IDE configuration.

    - Problems Encountered

n/a

  - 27/04/26: RMS Tolerance Percentage

    - Explanation

A comparison function was implemented to calculate percentage deviation between measured RMS values and nominal/reference values. This allows the system to determine whether phase values fall within acceptable tolerance limits, improving diagnostic capability.

    - Problems Encountered

n/a

# Testing

  - Test Inputs

The program was tested using the following types of inputs:

- A valid CSV file containing a full dataset of waveform samples (primary test case)

- A smaller trimmed version of the dataset to verify scaling behaviour

- Files with intentionally modified values (e.g., constant signal, noisy signal) to validate calculations

- Incorrect file paths to test error handling

- An empty CSV file to test edge behaviour

  - Non-existent File Handling

When a file path does not exist, the program fails at the file open stage. The file pointer returns NULL, which is checked before any reading occurs. If this happens, the program outputs an error message and terminates the loading process safely.

This prevents undefined behaviour such as attempting to read invalid memory or dereferencing a null pointer.

  - Empty File Behaviour

If an empty file is passed, the file opens successfully but no data is read. As a result:

- The dataset remains unpopulated

- Processing functions operate on zero entries

- RMS and other outputs may return zero or default values depending on initialisation

This highlighted the importance of validating dataset size before performing calculations to avoid misleading outputs or divide-by-zero conditions.

  - RMS Verification

RMS results were verified using a combination of methods:

- Manual calculation on a small known dataset (hand-calculated expected values)

- Cross-checking against Excel RMS formula results for identical data

- Testing with a constant signal (expected RMS equals the constant value)

- Testing with a sine-like waveform where RMS is expected to follow known theoretical relationships

These checks confirmed that the implementation correctly follows the RMS definition: Square each sample, then compute mean of squared values, then take square root of the result.

# Reflection

This project successfully developed a modular C-based system for importing, processing, and analysing power quality data from CSV files. The final implementation achieved key objectives including RMS calculation, peak-to-peak measurement, DC offset analysis, clipping detection, and tolerance checking.

One of the strongest aspects of the project was the gradual move toward modularity. Early versions were more monolithic, but restructuring into separate files (such as io.c, io.h, and waveform analysis modules) significantly improved readability and maintainability. This separation also made debugging easier, as each component could be tested in isolation.

Dynamic memory allocation was another important development step. Moving away from fixed-size arrays allowed the program to scale more effectively with different dataset sizes. However, this introduced additional responsibility in terms of memory management. Careful use of malloc and free was required to avoid leaks and ensure stable behaviour.

One of the biggest hurdles during development was understanding pointers and how memory is actually managed in C. Initially, pointer behaviour—especially when combined with dynamic memory allocation—was difficult to interpret, particularly around how arrays map to memory addresses and how data is passed between functions. Over time, working directly with allocated memory blocks and stepping through pointer arithmetic helped build a clearer understanding of how data is stored and accessed at a low level. This was a key learning point in the project.

A key challenge throughout the project was file handling and path management, particularly when working across different environments (IDE and OneDrive directory structure). This highlighted the importance of using consistent relative paths rather than absolute ones.

From an algorithmic perspective, RMS and signal processing calculations were straightforward but required careful validation. Verifying results against known values and external tools such as Excel was essential in confirming correctness. This step was critical, as numerical errors could otherwise go unnoticed.

There were also lessons learned around defensive programming. Early issues such as incorrect file inputs, empty datasets, and uninitialised variables showed the importance of input validation and safe defaults. Adding checks for file existence and dataset size improved robustness significantly.

If the project were extended further, potential improvements would include:

- More advanced statistical analysis (e.g. harmonics, FFT-based frequency analysis)

- Automated unit testing for signal functions

- Configuration files for threshold values instead of hardcoded constants

- Improved error reporting and logging system

Overall, the project demonstrates solid progression from a basic file parser to a structured signal analysis tool, with clear improvements in software design, memory management, and numerical validation throughout development.